Evaluating the efficiency and water recovery of direct contact membrane distillation (DCMD) systems with dual heat recovery and recirculation of retentate and permeate

Emerson B. dos Anjos1,*, Abdul O. Cárdenas Gómez1, Luz Elena P. Chenche1, João A. Lima1,2, Carolina P. Naveira-Cotta1, Renato M. Cotta1,3, Kleber M. Lisboa4

1Laboratory of Nano & Microfluidics and Microsystems - LabMEMS, Mechanical Eng. Dept., POLI & COPPE/UFRJ, Federal University of Rio de Janeiro/RJ, Brazil

2Renewable Energy Eng. Dept., DEER/CEAR, Federal University of Paraíba, João Pessoa/PB, Brazil

3 General Directorate of Nuclear and Technological Development, DGDNTM, Brazilian Navy, Rio de Janeiro/RJ, Brazil

4Laboratory of Thermal Sciences (LATERMO), Department of Mechanical Engineering - TEM/UFF, Universidade Federal Fluminense, Niterói/RJ, Brazil

*Corresponding author: emersonbanjos@coppe.ufrj.br

## ABSTRACT

A transient model for hollow-fiber direct contact membrane distillation (DCMD) modules is employed to evaluate a desalination plant in six modes of operation, namely batch, semi-batch, volume-control, salinity-control, continuous, and steady state. The main goal is to reveal DCMD desalination configurations that present the best compromise between maximum energy efficiency and minimum use of brackish water from wells, mainly focusing on applications in sites under hydrological stress, like in the semi-arid Brazilian northeastern region. The obtained results indicate that using an additional heat exchanger for heat regeneration, combined with recirculation of the retentate stream, enhances the system efficiency compared to a scenario that uses only the distillate heat recovery. Additionally, the results also showed that if the main interest is in obtaining high recovery ratios (high volume of produced distillate water per volume of brackish water), the recirculating semi-batch and volume-controlling operating modes are the best alternatives to produce distillated water. However, if the interest is to minimize the thermal energy cost, the best option is the batch operating mode.

Keywords: Desalination, DCMD system, heat recovery, retentate recirculation, energy efficiency.

Nomenclature

a	Specific surface area

cp	Averaged specific heat at constant pressure

C	Thermal capacity

Cs	Salt concentration

d	Diameter

D	Diffusivity/Diffusion coefficient

f	Friction factor

h	Convective heat transfer coefficient

hfg	Enthalpy/heat of vaporization

jwl	Distillate flow rate

k	Thermal conductivity

L	Hollow-fiber module length

m	Mass

m	Mass flow rate

M	Molar mass

N	Molar flux

Nfbr	Number of fibers in the hollow-fiber vessel

Nu	Nusselt number

p	Pressure

Pr	Prandtl number

qc	Heat transferred to the cold sink

qh	Heat transferred from the hot source

qHX	Heat transferred in the heat exchanger

Q	Rate of heat transfer

r	Radius

R	Universal gas constant

Re	Reynolds number

T	Temperature

V	Volume

V	Volumetric flow rate

u	Stream velocity

z	Position along the module

Greek symbols

α	Membrane effectiveness in retaining salt

γ	Relative to lumen or shell phase

δ	Membrane thickness

ε	Porosity

εHX	Heat exchanger efficacy

η	Efficiency

λ	Rejection fraction of the retentate stream

μ	Dynamic viscosity

ρ	Density

τ	Tortuosity

ϕ	Salinity

Subscripts/superscripts

a	Relative to air

amb	Relative to environment

col	Relative to collector tank

dist	Relative to distillate

e	Relative to inlet

ext	Relative to external

l	Relative to lumen

m/memb	Relative to membrane

mix	Relative to mixture

o	Relative to outlet

s	Relative to shell

vap	Relative to vapor

w	Relative to water

well	Relative to well

Indicates averaged value

Acronyms

AGMD	Air Gap Membrane Distillation

CGMD	Conductive Gap Membrane Distillation

DCMD 	Direct Contact Membrane Distillation

DLR	Distillate Limiting Ratio

GOR 	Gained Output Ratio

FLR	Feed Limiting Ratio

HICO	Hot Iin, Cold Out

HOCI	Hot Out, Cold In

HX	Heat Exchanger

MD	Membrane Distillation

PGMD	Permeate Gap Membrane Distillation

RO	Reverse Osmosis

RR	Recovery Ratio

SEC	Specific Energy Consumption

SGMD	Sweeping Gas Membrane Distillation

V-AGMD	Vacuum Air Gap Membrane Distillation

VMD	Vacuum Membrane Distillation

- Introduction

Freshwater scarcity, mainly due to population growth, has attracted the attention of researchers and industry for the development and improvement of techniques for the treatment, purification, and desalination of salt water. Among the main desalination techniques, those involving thermal separation methods, such as membrane distillation, require more energy due to the need to vaporize the water since the water vaporization enthalpy is about 2,400 kJ/kg (Sharqawy et al., 2010), while Gibbs free energy for seawater, which governs the energy use of the reverse osmosis method, is 2.7 kJ/kg (Elimelech and Phillip, 2011). Therefore, one of the challenges that must be overcome for the application of thermal methods such as membrane distillation (MD) in water desalination, which aims to be competitive against the conventionally employed reverse osmosis (RO) process in small and medium-sized systems (<100 m³/day of water produced), is to minimize the demand for thermal energy, improving thermal efficiency.

The membrane distillation process enables the use of residual heat from other thermal processes or the heat available from renewable sources, such as solar collectors. However, heat losses by conduction through the membrane further increase the thermal consumption, which, when added to high-water consumption, limits the large-scale use of these systems (Khayet and Matsuura, 2011). Nevertheless, according to Guan et al., (2015), when compared to conventional desalination processes, such as multi-stage flash distillation, multi-effect distillation, and reverse osmosis, the MD process has several benefits: low salinity sensitivity and high salt rejection (100% content, theoretically), low membrane vulnerability to fouling, greater performance under low severity operating conditions, and the viability of using low-grade waste heat from other thermal processes or renewable sources, such as solar collectors, as previously mentioned (Curcio and Drioli, 2005; Charcosset, 2009; Drioli et al., 2015).

Concerning its technology, according to the method of obtaining the vapor pressure gradient through the membrane, the MD is classified in the following configurations (Zhang et al., 2015; Swaminathan et al., 2018): DCMD (Direct Contact Membrane Distillation), AGMD (Air Gap Membrane Distillation), SGMD (Sweeping Gas Membrane Distillation), VMD (Vacuum Membrane Distillation), PGMD (Permeate Gap Membrane Distillation), CGMD (Conductive Gap Membrane Distillation), V-AGMD (Vacuum-Enhanced Air Gap Membrane Distillation), among other variants. Each configuration has its advantages and intrinsic disadvantages, so the choice of one of them is still a research theme, which is usually directed through metrics such as GOR (Gained Output Ratio) and SEC (Specific Energy Consumption), which are parameters for energy efficiency analysis. The DCMD is the most studied and straightforward configuration among them, widely used for desalination studies and concentration of aqueous solutions.

In general, the increase in the thermal efficiency of MD systems is achieved by improving the performance of membranes (by choosing membranes with low thermal conductivity, adequate porosity, pore size and pore size distribution, and thickness), as well as by reducing the amount of external heat to be supplied to the process (Gostoli and Sarti, 1998; Khayet et al., 2006; Al-Obaidani et al., 2008; Yao et al., 2013; Li et al., 2014; Swaminathan et al., 2018).

The heat supplied to the process can then be significantly reduced by the reuse of the heat available in the output currents of the desalination module through external heat exchangers, extending the applicability of the MD process and improving energy efficiency (Gilron et al., 2007; Lee et al., 2011; Hausmann et al., 2012; Lu and Chen, 2012; Zhang et al., 2015; Gonzalez-Bravo et al., 2015; Kim et al., 2015; Maheswari et al., 2015; Lin et al., 2014; Gustafson et al., 2018; Christie et al., 2020; Lisboa et al., 2021; Sampaio, 2022). Obviously, operating conditions such as temperature, the feed and distillate flow rates, salinity of the feed stream, and its recirculation, among others, significantly affect the MD process and can improve or reduce process efficiency (Khayet et al., 2000; Wang et al., 2009; Zhang et al., 2010; Fan and Peng, 2012, Alkhudhiri et al., 2012, Guan et al., 2014).

Additionally, since MD presents a low recovery ratio per pass (ratio between the production rate of pure water and the feed flow), using some form of recirculation of the brine/retentate stream is mandatory to increase efficiency. In this way, by considering a CGMD module, Swaminathan and Lienhard (2018) describe four operation strategies that enable high global recovery using a single-pass low recovery process (batch recirculation, semi-batch recirculation, continuous recirculation, and continuous multi-stage recirculation). The first two operations are classified as transient/discontinuous processes, while the last two are steady state ones. According to these authors, who studied the salinity variation from a lower limit of 70 g/kg to an upper limit of 260 g/kg, the batch operation is the best option since GOR is about twice for the same distillate flow when compared to the continuous recirculation operation. In their analyses, using a tank with a high volume to minimize salinity variation, they adopted a quasi-permanent approach, using a steady state MD model for instantaneous performance evaluation so that transient effects between each cycle could be ignored. In this respect, cycling time is thus considered to be very small.

Sampaio (2022) modeled a seawater desalination DCMD plant with heat recovery of the recirculating hot distillate stream by using a DCMD module composed by a bundle of hollow-fiber tubes. He combined analytical solutions for the mass transfer problem across the membranes with the finite volume method for solution the steady-state heat transfer problem. Still, based on Christie et al. (2020) and Lin et al. (2014), and to extend their works, Sampaio (2022) developed an approximation for the optimal value of the ratio between permeate and feed flow rates, and for the maximum mass recovery rate that, differently from the expressions developed by Christie et al. (2020) and Lin et al. (2014), take into consideration the membrane thermal efficiency.

Thus, based on the works of Swaminathan et al. (2018), Christie et al. (2020), Lisboa et al. (2021), and Sampaio (2022) and extending their analyses, the present work aims at the development of physical-mathematical modeling, in the unsteady-state regime, and subsequent numerical simulation of a hollow-fiber DCMD system with double recirculation and heat recovery. The main goal is firstly to evaluate the potential of heat recovery from the retentate stream to increase the energy efficiency of the system to produce distilled water. Furthermore, an application

of the model to study the desalination process in regions with high seasonal hydrological stress, such as the semi-arid region of the Brazilian northeastern. In addition to the classical batch, semi-batch, continuous, and steady-state modes of operation, two intermediary modes, namely, mixing tank volume controlling and mixing tank salt concentration controlling, are also introduced. The aim is to verify their effect on the behavior of the system efficiency and on the water recovery ratio (for minimizing the well brackish water consumption). These controlling schemes can also be operated in conjunction with total or partial recirculation, i.e., when the brine is rejected from the system with fraction rejections equal or different to the unity, respectively.

Table 1 presents, in a condensed way, a brief bibliographic review of the main experimental and simulation works on the thermal efficiency of MD systems, highlighting the main contributions of each of the works, especially, but not limited to, those works that adopt heat recovery and recirculation of flow streams using hollow fiber membrane configuration.

Table 1 - Literature review on thermal efficiency in MD systems by heat recovery.

| Reference | Heat Recovery | Recirculation |  | Observations |
| --- | --- | --- | --- | --- |
|  | Distillate | Distillate | Retentate |  |
| Fane et al. (1987) | X | X | X | Experimental/Simulation - Hollow fiber - DCMD |
| Schneider et al. (1988) | X | X | X | Experimental - Hollow Fiber - DCMD |
| Kurokawa and Sawa (1996) | X | X | X | Simulation - Flat fiber - DCMD |
| Li and Sirkar (2004) | --- | --- | --- | Experimental - Hollow Fiber - DCMD - Crossflow |
| Gilron et al. (2007) | X | X | X | Simulation - Hollow fiber - DCMD Cascade in Crossflow - Cascade of Exchangers |
| Lee et al. (2011) | X | X | X | Experimental/Simulation - Hollow Fiber - DCMD Cascade in Crossflow - 1 Changer |
| Hausmann et al. (2012) | --- | X | X | Experimental/Simulation - Flat Fiber - HX integrated into the DCMD Module for heat recovery |
| He et al. (2013) | X | X | X | Simulation - Hollow fiber - DCMD Cascade |
| Chung et al. (2014) | X | X | X | Experimental/Simulation - Hollow fiber - DCMD cascade in Crossflow - Recovery before/after retry rejection |
| Duong et al. (2015) | --- | X | X | Experimental/Simulation – Flat Fiber – DCMD – Optimization brine recycling |
| Lin et al. (2014) | X | X | --- | Simulation - DCMD - Critical parameter operation FLR/DLR |
| Guan et al. (2015) | X | X | X | Simulation - Hollow fiber - DCMD - Implicit expression for GOR |
| Zhang et al. (2015) | X | X | X | Review on heat recovery in MD systems |
| Lokare et al. (2018) | --- | X | X | Simulation - DCMD - Brine recycling - Impact on the electrical consumption |
| Christie et al. (2020) | X | X | X | Simulation - DCMD - Simplification FLR/DLR - Specific yield parameter: residual heat as the source |
| Lisboa et al. (2021) | X | --- | --- | Simulation - Hollow fiber - DCMD - Mathematical model porous medium |
| Sampaio (2022) | X | X | --- | Simulation - Hollow fiber - DCMD - New FLR/DLR relations |

DLR – Distillate Limiting Regime			FLR – Feed Limiting Regime

GOR – Gained Output Ratio			HX – Heat Exchanger

- System Description

The MD desalination plant modelled in the present work is characterized by a water desalination system that operates in the transient regime, containing a DCMD module (a vessel with a bundle of hollow fiber membranes), two tanks (a mixing and collector), and running with dual recirculation and dual heat recovery, where the heat available at the output of the module from the feed and distillate streams are recovered through the heat exchangers, HX-1 and HX-2, respectively, as shown in Figure 1. The heat source and sink are constant-temperature heat reservoirs, so the feed and distillate streams reach the specified values of the source/sink temperatures by supplying/extracting exactly the corresponding amount of heat required. This assumption is adopted for system evaluation purposes only since, in practice, with the use of heat exchangers, temperatures are not kept constant, but rather come from sensible waste heat source.

Figure 1 - Design of the DCMD system with double heat recovery and partial/total recirculation of the retry current.

Here, it is considered that at the initial time the feed circuit is completely filled with brackish water, which is at the well salinity (cswell) and ambient temperature (Tamb) conditions. The distillate circuit is filled with distilled water at the same ambient temperature. The feed stream, at the outlet of the mixing tank/reservoir at 3 (cs3,  V3=Vfeed and T3), enters the heat exchanger 1 (HX-1) which makes it hotter at 4 (cs4, V4 and T4), after receiving energy from the distillate stream coming from the output of the DCMD module at 11 (cs11, V11 and T11). As demonstrated in the literature, the mandatory use of this heat exchanger (HX-1) greatly increases the efficiency of the MD system by reducing the amount of heat (qh) that must be supplied by the heater to raise the supply current temperature to the working temperature at 5 (cs5, V5 and T5= Tfeed).

The concentrated module effluent (the retentate) in 6 (cs6, V6 and T6) can be removed from the system (as a reject stream) or it can be recycled (recirculated). In the present work, the computational code was built by taking into consideration both possibilities, simultaneously or not, depending on a flow rejection fraction (λ) in 9 (cs9, V9 and T9), which directly and dynamically impacts the concentration of the solution in the tank (which feeds the MD system) and its volume, Vmix. When λ=1, the brine is completely rejected in 9 (the MD system must be continuously fed from the well on a permanent basis). Otherwise, when λ≠<1, the retentate is partially recycled/rejected. Under this condition, unlike common practice, where a steady-state approach is employed for long duration operations, a fully transient operation/simulation must be adopted since the volume and concentration of the mixing tank vary dynamically with time.

When necessary, depending on salinity and volume of mixing tank 1 (or the operating regime), brackish water can be pumped from the well in 1 (with Vwell, cswell and Twell) passing through the heat exchanger 2 (HX-2) as a cold stream to gain energy carried by the effluent retentate from the DCMD module in 6, which will be cooled down to 7 (cs7, Q7 and T7). The stream at 7 can be directed, either totally (λ=0) or partially (0<λ<1), to tank 1 through the line at 8 (cs8, V8 and T8), to be mixed with the heated brackish water stream coming from 2 (cs2, V2 and T2).

Thus, in addition to the modes of operation of the desalination system (batch, semi-batch, volume or salinity controlling, continuous and steady-state), the numerical simulation to be developed will seek to understand the energy efficiency behavior when an additional heat exchanger, HX-2 (for thermal exchange between the retentate stream and the brackish water from the well), and partial rejection/recirculation of this retentate after thermal exchange can be employed.

To extend the scope of the simulation, a parametric analysis to evaluate the interaction between NaCl concentration of the feed fluid and the heat recovered from the fluid returning from the feed circuit of the desalination module, can also be accomplished. This also allows the development of operational control strategies to maximize the desalination efficiency while reducing the well water consumption.

To perform the present study, the operational ranges of the PLICA-7 DCMD module produced by Aquastill for experiments in a relevant environment are adopted. Thus, the computational code developed will seek to predict the situations that can happen in experiments with the pilot unit purchased from Aquastill, from small modifications in the installed system, or directly in the laboratory scale experimental module developed in LabMEMS/UFRJ.

- Mathematical Model

The implemented mathematical model incorporates heat and mass transfer in all system components: mixing tank, distillate tank, DCMD module, heat source and sink, and heat exchangers 1 and 2. For the DCMD module, the model proposed by Lisboa et al. (2021) was extended to the unsteady-state regime. The authors treated the hollow-fiber DCMD module as an equivalent porous medium composed of the following constituents: the porous shell medium (where the feed stream flows), the porous lumen medium (where the distillate stream flows), and the membrane (where the distillation process occurs). The membrane is modeled separately and is not included in the porous medium model. The counterflow configuration for the flow in the hollow fiber is adopted in the present work.

An illustrative figure of the porous media model employed to analyze the heat and mass transfer in the vessel containing the individual hollow fiber membrane modules, highlighting mainly the different constituents (lumen, shell/shell and membrane), is shown in Figure 2.

Figure 2 - Illustration of the porous media models. Cross sections of the: (a) Porous medium, lumen side; (b) Module and hollow-fiber membrane bundle; (c) Porous medium, shell side. Figure 1 from Lisboa et al. (2021). Reprinted with permission from Elsevier. Copyright 2021.

The hollow fiber membranes are considered to be uniformly distributed along a vessel (module) cross section, with their axis parallel to the vessel axis (which contains all the hollow fibers). The module porosity is defined as the ratio of the phase, either shell or lumen fluid, volume to the total vessel volume. For the two phases, we have:

|  | (1) |
| --- | --- |
|  | (2) |

Here, εγ is the porosity of the phase γ (with γ=l for the lumen phase and γ=s for the shell phase), Nfbr is the number of fibers in the vessel, rγ is the radius of the γ phase, and rv is the radius of the vessel. Given that the membrane has a thickness δ, the outer radius of the membrane will then be rs=rl+δ. In the present model, the shell radius, rc=rvNfbr, is an idealization in the simulations for the outer radius of a single membrane of the hollow-fiber module (Figure 3).

Figure 3 - Representation for the single-fiber model of the hollow-fiber membrane module.

The specific surface area, aγ, used in heat and mass transfer calculations, is the ratio of the surface area available for transport phenomena in each phase to the vessel volume.

|  | (3) |
| --- | --- |

The physical model for the control volume of the mixing tank considers two inlet streams (retentate stream (8), and brackish well water stream (2)) and the tank outlet stream (3). To recover the heat from the retentate stream, it would be possible to mix it with the well brackish water directly in the mixing tank, and in order to control the salinity in the tank, part of the mixture would be rejected immediately afterwards. This would be the most energetically efficient option, however, some of the brackish water from the well would then be rejected in this process, which is undesirable due to the hydrological stress in arid and semi-arid regions.

Therefore, to avoid this problem, and still recover a large part of the thermal potential of the circulation stream, it is proposed to use an additional heat exchanger (HX-2) before the rejection process (as illustrated in Figure 1), which can be partial or not. Thus, the mixing tank (and therefore all the desalination system) will be a dynamic system, since the volume, Vmix, the concentration, csmix (or salinity, ϕmix) and the temperature, Tmix, of the mixture inside it change over time.

Considering that the heat exchangers, the distillate and mixing tanks, and the DCMD module are adiabatic; and that the salinity and temperature fields inside them are uniform (except inside the DCMD module), the physical situation is mathematically modeled as:

- Mixture and Distillate Tanks

|  |  | (4) |
| --- | --- | --- |
|  |  | (5) |
|  |  | (6) |
|  | (7) |  |  |

- DCMD Module:

|  | (8) |
| --- | --- |
|  | (9) |

Equations (4-9) are subjected to the following initial conditions:

|  | (10) |
| --- | --- |

Additionally, since equations (8) and (9) are partial differential equations, they are subject to the following boundary conditions:

|  | (11) |
| --- | --- |
|  | (12) |

In these expressions, ργ is the specific mass of phase γ, cpγ is the specific heat at constant pressure of phase γ, kγ is the thermal conductivity of phase γ, Tγ is the bulk temperature of phase γ, Tmγ is the interface temperature between the hollow fiber membrane surface and the liquid phase γ, and hγ is the convection heat transfer coefficient at the interface between the liquid phase γ and the membrane. The convective heat transfer coefficients for the hot and cold streams are evaluated through the following empirical correlations:

|  | (13) |
| --- | --- |

Where Reγ is the Reynolds number, based on the hydraulic diameter, dhγ, Prγ , the Prandtl number and fγ is the friction factor, all related for the γ-phase, which are defined as:

| ; | (14, 15) |  |
| --- | --- | --- |
| ; |  | (16, 17) |  |

These equations are also coupled to the additional equations of the mass and energy balances for the remaining control volumes in the feed and distillate streams. Thus, the following relationships are obtained:

- For the temperatures

| (input) , |  | (18, 19) |  |
| --- | --- | --- | --- |
| , |  |  | (20, 21) |
| (input) , | (22, 23) |  |  |
|  | (24) |  |  |
| (input) , | (25, 26) |  |  |
|  | (27) |  |  |

- For the flow rates

| (input) , | (28, 29) |
| --- | --- |
| , | (30, 31) |
| , | (32, 33) |
| , | (34, 35) |
| , | (36, 37) |

- For the salinities

| (input) , | (38, 39) |
| --- | --- |
| , (input) | (40, 41) |
|  | (42) |

The equations for the temperatures (T4, T12) and (T2, T7) are obtained from the definitions of the effectiveness of heat exchangers HX-1 and HX-2, respectively:

|  | (43) |
| --- | --- |
|  | (44) |
| , | (45, 46) |

In the salinity equations, αmemb is the effectiveness of the membrane in retaining salt (not letting the salt to pass into the distillate stream), taken as 100% in the present study (αmemb=1). The properties of the brackish well water are determined in each point of the system and are a function of the temperature and salinity of the point analyzed (Sparrow, 2003).

The temperatures of the membrane interfaces along the z-direction of the DCMD module, Tm,l(z,t) and Tm,s(z,t), required in Eqs. (8) and (9), are evaluated iteratively and dynamically through a procedure within the routine that solves the system of differential equations, by equating the local heat transfer rates of the feed, membrane, and distillate regions:

|  | (47) |
| --- | --- |
| , | (48-50) |
|  | (51) |
|  | (52) |
|  | (53) |

The density of the mixture at each position or control volume of the circuit, ρi, is related to the salinity, ϕi, through the density of pure water and salt at Tamb=25 °C (ρsalt=2,165 kg/m3 and ρH2O=997.4 kg/m3) as:

|  | (54) |
| --- | --- |

Other fluid properties, such as specific heat at constant pressure, thermal conductivity, viscosity, and enthalpy of vaporization, are also evaluated as a function of salinity and temperature, according to the correlations proposed by Sparrow (2003), or by interpolation between values from thermophysical property tables. The model for the membrane effective thermal conductivity, km, constituted by the polymer matrix and the water vapor inside its pores, will be discussed later.

3.1 Modeling Mass Transfer in the Membrane

To partially close the system of equations, the mass transfer inside the DCMD module must be modeled in order to determine the water vapor mass flux through the membrane, jwl. For this, as performed by Lisboa et al. (2021), the Dusty Gas model (Lawson and Lloyd, 1997), one of the most widely used models for predicting flow of distilled water through membranes, was employed.

|  | (55) |
| --- | --- |

where Nw is the molar flux of water vapor, Na is the molar flux of air, pa is the pore partial pressure of air, pw is the pore partial pressure of water vapor, Dwk is the Knudsen diffusivity of water vapor, Dwa0 is the molecular diffusivity of water vapor in air, R is the universal gas constant, and Tm is the thermodynamic average temperature at each axial position along the membrane.

Air is normally considered at rest in the membrane pores, so that Na≅ 0. Applying Dalton's law, one obtains the following expressions for the molar, Nwl (jwl, and mass, jwl, fluxes of distilled water through the hollow fiber membranes (based on rl):

| , | (56, 57) |
| --- | --- |

The Knudsen diffusivity of water vapor, Dwk, the molecular diffusivity of water in air, Dwa0, and the effective molecular diffusivity, Deff, are evaluated using the classical relationships:

| , , | (58-60) |
| --- | --- |

The total pressure inside the membrane pores, P, is usually the atmospheric pressure. The partial vapor pressures of the salt streams at the membrane interfaces, ps and pl, are evaluated from the saturated vapor pressures, pvγ, obtained by using the Antoine equation, corrected for the respective salt mole fractions (xsalt,γ) and activity coefficients (αγ):

| , | (61, 62) |
| --- | --- |

The morphological characteristics of the membrane employed in the module are taken into account in the calculation of the distillate flux through the membrane mean pore diameter, dp, porosity, εm, and tortuosity, τm, as can be seen from these equations.

The membrane effective thermal conductivity, km, and tortuosity, τkm, models must be selected to close the governing equations. The most common models for the membrane thermal conductivity are based on geometrical distributions of the solid and gaseous phases. In the isostrain (parallel) model, the solid and gaseous phases are disposed in parallel. The isostress (series) model considers that phases are arranged in series. García-Payo and Izquierdo-Gil (2004) recommend that, for porous media with porosity greater than 0.6, the Maxwell I thermal conductivity model be used, which considers that the polymeric matrix can be thought of as a random distribution of solid particles that do not interact themselves in a homogeneous medium (air). These models are mathematically described in terms of the solid matrix, ks, and fluid/gaseous, kf, conductivities, as:

| , parallel model | (63) |
| --- | --- |
| , series model | (64) |
| , , Maxwell I model | (65, 66) |

Additionally, the way as pores and the solid matrix are distributed inside the membrane governs the path for vapor diffusion and, therefore, its flux. Hence, membrane tortuosity, defined as the ratio between the real path for diffusion of a species and the membrane thickness, is an important parameter for the prediction of the mass flux through the membrane.

A significant number of tortuosity models can be found in the literature (Ghanbarian et al., 2013). Recently, Kim et al. (2021) analyzed 11 models of tortuosity: eight models are supported on geometric hypothesis, based on the distribution of Euclidian dimensions (cubes, spheres and cylinders) in the porous structure of the membrane; and three models are supported on fractal theory of porous medium, which assumes that the fluid molecules diffuse inside a fractal system. In general, either in Euclidian or fractal models, the tortuosity is related to the membrane porosity, as for example:

Geometric (Euclidian) Models:

| , Mackies and Meares (1955) | (67) |
| --- | --- |
| , Beeckman (1990) | (68) |
| Fractal Models: |  |
| , Elias-Kohav et al. (1991) | (69) |
| , Bruggeman (1935) | (70) |

According to Curcino et al. (2022), that analyzed some effective thermal conductivity and several tortuosity models in their simulations of MD in flat and hollow-fiber membrane configurations, an adequate selection must be supported on the knowledge of the morphological and structural properties of the membrane used in the experimental desalination module.

  - Efficiency metrics

To evaluate the desalination system and verify the improvement in energy efficiency in the membrane distillation system, the thermal efficiency, , the gain-output ratio, GOR, the specific thermal energy consumption, SECth, and the recovery ratio, RR, are employed and analyzed in different operational scenarios. Those quantities are defined as:

| , , , | (71-74) |
| --- | --- |
| where, |  |
|  | (75) |
|  | (76) |
|  | (77) |
| , | (78, 79) |
|  | (80) |
|  | (81) |

The global GOR and SECth parameters are taken as the average values up to the simulation end time. The volumes of produced distilled water and consumed well brackish water are obtained from the integration of the instantaneous volumetric flow rates.

A computational code to solve the coupled system of ordinary and partial differential equations for the temperature, flow rates, and salinities was developed in the Fortran 95 language. The MMOLCH subroutine (IMSL, 2022), for solving hyperbolic partial differential equation systems through the method of lines, was employed, with a tolerance error set as 10-5. The non-linear equations for the interface temperatures Tms and Tml were solved, within an error target prescribed at 10-11, with the routine ZBREN (IMSL, 2022).

- The system operating modes

To control the operation of the dynamic system and optimize the desalination system water production and brackish water consumption, a general algorithm based on volume and salinity of the mixture in the mixing reservoir, which feeds the desalination system, is applied at each instant of time. The developed computational code is very flexible, allowing simulations of different desalination system operation scenarios: steady-state regime (no recirculation), batch recirculation, semi-batch recirculation, continuous recirculation (dynamic rejection fraction/recirculation), and partial recirculation operation mode with volume or concentration control.

For the unsteady-state regimes, the system can start the operation with no brine rejection (total recirculation), λ=λ0=0, or with a pre-defined rejection (partial recirculation), λ0≠0. For these regimes, the desalination system operates in such a way that the mixing tank, initially with a volume of brackish water of Vmix0=100 L, has its mixture consumed by the DCMD module, as distillate is produced over time.

Additionally, to analyze the influence of the external heat exchangers on the system performance it is sufficient to vary their effectiveness (εHX1 and εHX2). By choosing εHX1= 0 and/or εHX2= 0, the heat exchangers are not included in the analysis.

4.1 Batch and semi-batch operations

In batch and semi-batch operation modes, over each process cycle, the distillate is continuously removed from the system, and the remaining solution that leaves the MD module, with higher salinity, is directed to the feed tank (λ=0). In the batch operation process, there is no water pumping from the well (Vwell=0) between recycling, so the volume of the solution in the tank decreases, and the salt concentration increases over time. In the semi-batch operation cycle, the volume of the mixing tank is kept constant, by continuously adding fresh feed water from the well to the system, at the same proportion the distillate is removed (Vwell=Vjwl).

In both operation processes, the permeate production rate, as well as the heat transfer rate, will reduce as the feed stream of the MD module becomes increasingly saturated with salts. After the salinity of the solution in the mixing tank reaches a limiting value, the high-salinity brine is flushed-out (λ is set to 1) until a minimum volume allowed is reached. Then, the power supply refills the tank from the well (Vwell=Vs) until the minimum concentration is reached, and the cycle restarts. During these processes (flushing and filling), the HX-2 should be used to improve the system’s efficiency.

4.2 Continuous operation (steady-state operation with partial brine recirculation)

In the continuous operation regime, a fixed operational brine salinity is specified at the MD module exit. Therefore, after the initial transients have elapsed and the solution that leaves the module is at the required salinity (reaching the steady-state regime), it must be continuously bled-out of the system at a specific fraction rejection (λ) that depends on the MD module performance. The make-up water from the well (Vwell) is simultaneously added to the tank at a proportional flow rate that keeps constant the salinity of the stream at the MD module exit, which is related to that fraction rejection. Again, the HX-2 should be used to improve the system’s efficiency.

The rejection fraction and the volumetric flow rate from the well is obtained according to the following salt and mixture mass balances at the mixing tank:

|  | (82) |
| --- | --- |
|  | (83) |

4.3 Steady-state operation with no brine recirculation

To simulate a steady state condition with no recirculation, it is sufficient to set λ=λ0=λ1=1 and Vwell=Vs (well flow rate equal to the nominal operating flow rate of the DCMD module) all the time during the simulations. In this situation, all retentate leaving the DCMD module is rejected and the system is continuously fed by the well at constant feed flow rate and salinity. As will be showed, if this mode of operation is mandatory (as is the case studied by Sampaio, 2022), use of the second heat exchanger (HX-2) to recover the heat from the retentate stream before it is flushed-oud will lead to an improvement in the performance of the desalination system.

  - Volume or Concentration controlling

Since these modes of operation are of the transient type, the distillate is continuously removed from the system, and the remaining solution that leaves the MD module, with increasing salinity, is directed to the feed tank or is rejected (it depends on the value of λ). In one case or another, use of the HX-2 will improve energy efficiency, as will be demonstrate later.

In the mixing tank volume controlling operation, a minimum and a maximum allowed mixing tank volume is set as the operation limits. The salinity of the water in the mixing tank (csmixt) is continuously increasing with the distillate production, and its volume (Vmixt) decreases. When the minimum volume is reached (Vmin), the tank is filled with brackish water from the well up to the maximum volume (Vmax) previously chosen. The process then repeats until finishing the desalination operation. Eventually, in the case the mixture in the mixing tank reaches a maximum operation salinity (also previously set), it is completely flushed-out (λ=1) and, after this, the tank is refilled with brackish water from the well up to the initial volume, Vmix_0.

On the other hand, in the mixing tank salinity controlling operation, a minimum and a maximum allowed mixing tank salinity is set as the operation limits. The salinity of the water in the mixing tank (csmixt) is continuously increasing with the distillate production, while its volume (Vmixt) decreases since there is no brackish water flow from the well. At the instant the maximum salinity allowed is reached (Csmax), the pump from the well is turned on to feed the mixing tank until the salinity of the water contained in it falls to the minimum predefined value. In this process, the volume of the mixing tank is increased, so its operational range will be dynamically adjusted according to the values of salinity limits previously set.

- Results

5.1 Algorithm verification

Before analyzing typical well operation scenarios, the developed computational code will be verified by comparing its results with available literature data. Since no transient results are available, these comparisons will be made with data for the steady-state regime.

Table 2 presents the physical and geometric characteristics of the membranes and modules, such as material, porosity, thickness, pore diameter, and thermal conductivity, employed in the comparisons. The experimental results of Wang et al. (2008), Yang et al. (2011) and numerical results of Guan et al. (2014) and Kim (2014) are employed for situations with no heat exchangers, i.e., only the performance of the DCMD module is studied.

Table 2. Characteristics (input data) of the membranes and modules used for comparison.

| Parameter | Symbol | Unit | Wang et al. (2008) | Kim (2014) | Yang et al. (2011) | Guan et al. (2014) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Experimental | Numerical | Experimental | Numerical |
| Membrane inner radius | rl | mm | 0.3 | 0.3 | 0.46 | 0.49 |
| Membrane thickness | t | mm | 0.11 | 0.11 | 0.275 | 0.240 |
| Polymer thermal conductivity | kp | W/(m.K) | 0.18 a | 0.18 | 0.18 a | 0.18 a |
| Average pore diameter | dp | mm | 0.17 b | 0.17 | 0.145/0.191 | 0.2 b |
| Porosity | m | --- | 0.738 | 0.738 | 0.83 | 0.85 b |
| Shell inlet temperature | Te_s | oC | 30.0-79.3 | 30-79.3 | 35-70 | 30-80 |
| Lumen inlet temperature | Te_l | oC | 17.5 | 17.5 | 25 | 23.5-30 |
| Total pore pressure | P | kPa | 101.325 a | 101.325 a | 101.325 a | 101.325 a |
| Shell side flow rate | Vs | L/min | 1.64768 c | 1.5209 | 2.5-3.0 | 1.1425-3.0 |
| Lumen side flow rate | Vl | L/min | 0.40715 c | 0.4072 | 0.4 | 0.04635-0.4 |
| Number of fibers | Nfbr | --- | 30 a | 30 | 20 | 22 |
| Vessel radius | rv | mm | 3.2413346 a | 3.2316 | 4.75 | 7.5 |
| Vessel length | L | m | 0.200 | 0.200 | 0.225 | 0.200 |
| Salt mass fraction shell | wNaCls | [%] | 3.5 d | 3.5 | 3.5 | 7.0 |
| Lumen salt mass fraction | wNaCll | [%] | 0.0 | 0.0 | 0.0 | 0.0 |
| Cold/hot fluid arrangement | --- | --- | HOCI | HOCI | HOCI | HICO |
| Direction of flow | --- | --- | Co-current | Countercurrent | Countercurrent | Countercurrent |
| Membrane area | Amemb | cm2 | 150 | 152.7 | 220 | 180.9 |
| Packing density | φ | [%] | 48 a | 48 | 50 | 20.8 a |

(a)	Obtained from, or employed the calculation procedure in, Kim (2014).

(b)	Assumed in comparison with other works.

(c)	Calculated as a function of average velocities (us = 1.6 m/s and ul = 0.8 m/s) and the respective transversal section areas.

(d)	Lisboa et al. (2021) used wNaCls= 2.5 % in their simulations.

It is worth mentioning that the experiments by Wang et al. (2008) were performed for the concurrent flow arrangement, distinct from the counter-current one adopted in the present work. Notwithstanding, the numerical results of Kim (2014) who studied the two types of arrangement show that, for the range of parameters studied, the differences in results are small.

It is also worth pointing out that, although obtained for the counter-current flow arrangement, the numerical results of Guan et al. (2014) adopted the configuration of cold fluid in the shell and hot fluid in the lumen (HICO), while the present work uses the HOCI configuration (hot fluid in the shell and cold fluid in the lumen). According to Guan et al. (2012, 2014), supported on the conclusions of Wirth and Cabassud (2002), there are no significant differences between the HICO and HOCI configurations when similar hydrodynamic conditions are maintained in the two streams.

Table 3 presents a verification of the results obtained with the present work together with the work of Lisboa et al. (2021) and a comparison with the experimental results of Wang et al. (2008), for the outlet temperatures of the feed and distillate streams, as well as the distillate flow.

Table 3. Verification of the computational code with the numerical results of Lisboa et al. (2021) and comparison with the experimental data of Wang et al. (2008). Te_l = 17.5 °C.

| Te_s [oC] | ∆T (Te_s - Te_l) [oC] | To_s [oC] |  |  | To_l [oC] |  |  | jwl* [kg/(m2.h)] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Present, and Lisboa et al. (2021) | Wang et al. (2008) |  | Present, and Lisboa et al. (2021) | Wang et al. (2008) |  | Present | Wang et al. (2008) |
| 40.0 | 22.5 | 38.5 | 39.0 |  | 23.4 | 24.5 |  | 5.5 | 6.2 ± 0.4 |
| 49.5 | 23.0 | 47.2 | 48.0 |  | 26.5 | 27.8 |  | 10.1 | 11.5 ± 0.7 |
| 59.8 | 42.3 | 56.2 | 57.4 |  | 31.7 | 31.3 |  | 20.0 | 19.9 ± 1.0 |
| 70.2 | 52.7 | 65.1 | 67.2 |  | 37.5 | 36.2 |  | 32.8 | 30.8 ± 1.0 |
| 79.3 | 61.8 | 72.6 | 76.0 |  | 43.8 | 39.0 |  | 47.9 | 41.5 ± 1.2 |

* Based on the outer radius of the hollow fiber

Twang_et_al. (2008): T ± 0.1 oC

The results show that the present methodology is consistent with the analyzed physics, at least in the steady-state regime, when compared to the experimental results of Wang et al. (2008). This table also shows that the results obtained with the methodology in transient regime converge, as would be expected for long times, to the numerical results in of Lisboa et al. (2021), who also verified their code with the experiments of Wang et al. (2008).

Another verification is illustrated in Figure 4, which shows a comparison of the results obtained in the present work for the produced distillate flux (which match the numerical results obtained by Lisboa et al. (2021)) with the experimental results of Wang et al. (2008) and the numerical results of Kim (2014).

The comparison with the results of Wang et al. (2008) is made by employing the parallel model of effective thermal conductivity (Eq. 63) and the fractal model of Elias-Kohav et al. (1991) for tortuosity (Eq. 69), while the comparison with the results of Kim (2014) is made with the Maxwell I model (Eq. 65) of effective thermal conductivity and the model of Beeckman (1990) for tortuosity (Eq. 68). The distillate mass flux was calculated with the surface area based on the outer radius of the hollow fiber.

Figure 4 - Comparison between the result obtained with the present methodology and the experimental results of Wang et al. (2008) and numerical results of Kim (2014) for the distillate flux.

To illustrate the spatial temperature distribution along the DCMD module, Figure 5 brings a comparison between the results obtained with the present formulation and the numerical results of Guan et al. (2014) for the steady-state temperatures along the feed channel, Tsz,∞, of the feed/membrane interface, Tmsz,∞, of the membrane/distillate interface, Tmlz,∞, and of the distillate channel, Tlz,∞. The inlet temperatures and flow rates of the feed and distillate streams employed in the simulation were: Te_s= 80 °C, Te_l= 30 °C and Vs= 1.1425 L/min,  Vl= 0.04635 L/min, respectively.

Figure 5 - Temperature distribution along the axial position in the DCMD module in the feed, interface, and distillate regions. Comparison with the numerical results of Guan et al. (2014).

Although the effective thermal conductivity and tortuosity models employed by Guan et al. (2014) were not defined in their work, the agreement between the numerical results is satisfactory. Here, the results were obtained by making use of the parallel model for the effective thermal conductivity and the fractal model of Elias-Kohav et al. (1991) for the tortuosity. In addition to these differences, the HICO flow configuration used in the work of those authors differs from the HOCI configuration simulated in the present work, so the small departures from their results were expected.

5.2 Simulations with the brackish water well

5.2.1 Preliminary Analysis

For simulation of a real desalination system fed by brackish water wells, data from wells commonly found in the northeastern region of Brazil were used, whose temperatures, salt concentrations, typical flow rates, and other properties can be found in Veiga (2017). Table 4 provides the characteristic parameter values, which serve as input data for the code, of 444 wells located in the Ceará state of Brazil.

The parameters relating to the characteristics of the DCMD module and the membrane are those provided in Table 5. The flow arrangement of the module are carried out in the HOCI configuration (hot fluid in the shell and cold fluid in the lumen) in counter-current.

Table 4. Characteristics of wells in the state of Ceará/Brazil.

| Limits | Salt concentration | Flow rates |  |
| --- | --- | --- | --- |
|  | [gs/L] | [m3/h] | [L/min] |
| Maximum | 13 | 15 | 250 |
| Minimum | 0.199 | 0.10 | 1.67 |
| Average | 2.532 | 3.45 | 57.5 |
| Adopted | 3.5 | 3 | 50 |

Twell = Tamb=25 oC

The data of the module was obtained from the work of Lisboa et al. (2021), who performed steady-state analyses of a DCMD module with membrane surface area similar to that manufactured by the company Aquastil (PLICA 7 model, membrane area of 7.2 m2), and installed in the external area of the LabMEMS/UFRJ. Additional parameters were evaluated using the procedure illustrated in Kim (2014) and, based on literature data, adopting values of characteristic membrane parameters (porosity, thickness, among others). All the results presented were obtained by making use of the parallel model for the effective thermal conductivity and the fractal model of Elias-Kohav et al. (1991) for the tortuosity.

Table 5. Input data, membrane and module, used in the simulation of the wells.

| Parameter | Symbol | Unit | Lisboa et al. (2021) |
| --- | --- | --- | --- |
| Membrane inner radius | rl | mm | 0.3 |
| Membrane thickness | t | mm | 0.1 |
| Polymer thermal conductivity | kp | W/(m.K) | 0.2 |
| Average pore diameter | dp | mm | 0.2 |
| Membrane porosity | m | --- | 0.8 |
| Inlet temperature on shell side | Te_s | oC | 80 |
| Inlet temperature on the lumen side | Te_l | oC | 25 |
| Total pore pressure | P | kPa | 101.325 |
| Shell-side flow rate | Vs | L/min | 30-50-80 |
| Flow rate on the lumen side | Vl | L/min | 30-50-80 |
| Number of fibers | Nfbr | --- | 10,000 |
| Vessel radius | rv | mm | 50 |
| Vessel length | L | m | 1 |
| Salt mass fraction on shell side | wNaCls | [%] | 0.035 |
| Lumen-side salt mass fraction | wNaCll | [%] | 0 |
| Membrane area | Amemb | m2 | 7.2 |
| Packing | φ | [%] | 0.64 |

Simulations were performed for various scenarios, all of them starting from the condition that the well feeds the system through the mixing tank (initially Vmix_0=100 L and Tmix=25 oC) only when some of the points of the controlling operating mode adopted are reached.

To illustrate a typical dynamic behavior of the heat transfer process inside the DCMD module, Fig. 6 shows the (bulk) temperature profiles of the retentate and distillate streams along the respective channels inside the module, at four instants of time (approximately 2.8x10-4 s, 3.4 s, 16 s, and 30 s), for the flow condition Vl = Vs=50 L/min. The simulation was performed with no recirculation of the brine (λ=1), and use of heat exchangers is irrelevant in this type of analysis.

Figure 6 - Time evolution of temperature profiles inside the DCMD module for Vl = Vs=50 L/min.

As can be seen from this figure, the time evolution of the heat transfer inside the DCMD module is very fast, since the steady-state condition is reached in just a few seconds, taking no more than 30 s for the module to attain this regime. Therefore, any numerical (experimental) analysis employing large time steps (measurement intervals) will not adequately capture the transient phenomena happening inside the DCMD module.

Figures 7a and 7b illustrate bulk temperature profiles of the feed and permeate channels along the module for different conditions of volumetric flow rate of the streams, considering steady-state single-pass flow in the DCMD module, and no heat recovery, i.e., λ0 =  λ = 1, Vwell = Vs, εHX1= εHX2= 0. The objective is to obtain indications of possible flow configurations (well and distillate flow rates) with adequate potential for using heat exchangers to enhance the energy efficiency of the desalination system, as well as to optimize the water recovery (maximize the distillate flux).

Figure 7a illustrates two scenarios where the distillate flow rate is lower than the feed flow rate, and two scenarios where the distillate flow rate is greater than the feed flow rate, approximating what Christie et al. (2020) called the DLR (Distillate Limiting Regime) and the FLR (Feed Limiting Regime) configurations, respectively. Figure 7b shows three cases where the distillate and feed flow rates are equal.

(a)						     (b)

Figure 7 – Steady-state temperature profiles along the axial position in the module DCMD for:
a) different feed and distillate flow rates, b) equal distillate and feed flow rates (Vl = Vs).

At a first glance, observing the streams outlet temperatures of the DLR configuration on Figs. 7a, one could point out that this configuration would have a great potential for energy recovery, since both streams present the highest outlet temperatures (79.6 oC for the distillate stream, and 58.8 oC for the feed stream), while in the FLR configuration only the distillate stream presents potential for heat recovery (44.8 oC). However, as it will be shown, the DLR configuration presents the lowest water recovery factor and the highest specific thermal energy cost, becoming inappropriate for heat regeneration.

For an intuitive understanding of how the operational and configurational decisions affect the energy efficient, Christie et al. (2020), based on Lin et al. (2014), introduced a critical mass flow rate ratio, α*, to define the theoretical condition of maximum water recovery factor in steady-state single-pass DCMD systems, Rmax, by comparing the critical (α*) and actual (α) mass flow rate ratios (by adopting equal heat capacity flow, mscps=mlcpl):

| , | (84, 85) |
| --- | --- |
| , | (86, 87) |

For α<α*, the module operates in the DLR regime, i.e., the distillate flow rate is smaller than the feed flow rate, so the distillate temperature rises in the module to approach the feed threshold temperature, Tthres_s, define as the temperature at which the feed stream at a temperature andTe_s concentration respectively at Te_s Cs has the same partial vapor pressure as the distillate stream (Cs=0) at temperature Tthres_s, i.e., when the trans-membrane vapor pressure difference vanishes, pv(Cs,Te_s)=pv(0,Tthres_s). Conversely, for α>α*, the module operates in the FLR regime, i.e., the feed flow rate is smaller than the distillate flow rate, so the feed temperature drops in the module to approach the distillate threshold temperature, Tthres_l, that is, the temperature at which the distillate stream (Cs=0) at temperature Te_l has the same partial vapor pressure as the feed stream at temperature Tthres_l and concentration Cs, i.e., pv(Cs,Tthres_l)=pv(0,Te_l).

When the temperature difference at all positions along the MD module is always greater than the threshold temperature difference, and the vapor flux occurs throughout the entire membrane length, only the mass transfer would be limited (MTLR) - insufficient membrane area or very low mass transfer coefficient. Then, if the mass transfer is not limited, a DCMD module always works in the thermodynamic limiting regimes of DLR or FLR.

In the words of Christie et al. (2020), for single-pass counter-current flow in DCMD desalination, the FLR configuration would be the most appropriate operational condition for water recovery, because only in FLR does the distillate stream have enough heat capacity flow to ensure that the minimum evaporation of the feed stream can be achieved, so RmaxFLR>RmaxDLR.

Table 6 brings some results obtained for the averaged theoretical maximum water recovery factor, distillate mass flux, temperature polarization coefficient, and DCMD module thermal efficiency (GOR, and SECth). Three scenarios related to the use of the external heat exchangers for two feed salinity levels and different feed and distillate flow rates were studied: without HX (εHX1=εHX2=0.0), HX-1 only (εHX1=0.8, εHX2=0.0),  HX-1 and HX-2 (εHX1=0.8, εHX2=0.8).

The results presented in Table 6 agree with (Christie et al., 2020) statements, showing that the best operation conditions are those FLR configurations to which α≅α* (where no threshold temperature is attained). In these configurations, high recovery factor (RR) and the lowest specific thermal energy cost (SECth) are attained. For the DLR condition, α<α*, decreasing the flow rate ratio α, reduces the water recovery factor and increases the specific thermal energy cost, making this configuration unfeasible, even with the reduction in energy costs brought about by the use of the second heat exchanger (HX-2). On the other hand, for those FLR situations to which α≫α*, the highest water recovery is reached at the expense of the highest energy costs though. Besides this, for this condition, use of the second external heat exchanger (HX-2) does not bring any substantial improvement to the DCMD system.

Table 6 also reveals that, for increasing brackish water salinity, by using flow rate ratios α slightly higher than α* (FLR) can improve the recovery factor (RR), distillate flow rate (Vjwl) and thermal efficiency (ηth) at the expense of only a small increase of SECth.

5.2.2 Operating Regime Analysis

After characterizing the DCMD desalination system, investigation of the behavior of the system for different operating modes is now performed. In all situations analyzed, the FLR configuration to which Vl = Vs = 50 L/min is adopted. Where not explicit stated, the two external heat exchangers, HX-1 and HX-2, are used with the following effectiveness: εHX1=0.8, εHX2=0.8.

For a better understanding on whether using the external heat exchangers brings an enhancement of the energy efficiency, Figures 8 and 9 illustrate the dynamic behavior of the GOR and SECth metrics for the batch operation mode and for two different maximum allowed salt concentration of the brine in the mixing tank analyzed, Csmax (120 gs/Lmix and 260 gs/Lmix, respectively). The averaged values indicated in these figures (GOR, SECth) were obtained for an operating time of 8 hours of the DCMD system.

Table 6. Influence of the operational conditions on the averaged DCMD module efficiency metrics. Steady-state regime (8h).

| Flow Condition [L/min] | α | α* | Oper. | Tthr [oC] | Zthr [m] | RR | Vjwl [L/min] | TPC [-] | ηth [%] | GOR [-] |  |  | SECth [kWh/L] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | εHX1=0.0 εHX2=0.0 | εHX1=0.8 εHX2=0.0 | εHX1=0.8 εHX2=0.8 | εHX1=0.0 εHX2=0.0 | εHX1=0.8 εHX2=0.0 | εHX1=0.8 εHX2=0.8 |
| wNaCls=3.5%≅35 gs/Lmix |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vl=30 < Vs=80 | 0.38 | 0.88 | DLR | 80.0 a | 0.63 | 2.5 | 2.6 | 0.73 | 73 | 0.28 | 0.41 | 1.4 | 2.5 | 1.7 | 0.48 |
| Vl=30 < Vs=50 | 0.60 | 0.88 | DLR | 80.0 a | 0.98 | 3.7 | 2.5 | 0.62 | 69 | 0.43 | 0.90 | 2.4 | 1.6 | 0.78 | 0.29 |
| Vl=50 < Vs=80 | 0.63 | 0.88 | DLR | --- | --- | 4.0 | 4.2 | 0.50 | 71 | 0.45 | 0.97 | 2.5 | 1.6 | 0.72 | 0.28 |
| Vl≅44 < Vs=50 | 0.88 | 0.88 | FLR | --- | --- | 5.0 | 3.3 | 0.56 | 67 | 0.56 | 1.9 | 3.3 | 1.2 | 0.36 | 0.21 |
| Vl=Vs=30 | 1.0 | 0.88 | FLR | --- | --- | 5.3 | 2.1 | 0.61 | 54 | 0.60 | 2.1 | 2.6 | 1.2 | 0.33 | 0.27 |
| Vl=Vs=50 | 1.0 | 0.88 | FLR | --- | --- | 5.2 | 3.4 | 0.59 | 66 | 0.58 | 1.8 | 2.5 | 1.2 | 0.38 | 0.27 |
| Vl=Vs=80 | 1.0 | 0.88 | FLR | --- | --- | 4.9 | 5.2 | 0.58 | 67 | 0.55 | 1.5 | 2.4 | 1.3 | 0.46 | 0.29 |
| Vl=80 < Vs=50 | 1.6 | 0.88 | FLR | --- | --- | 5.3 | 3.5 | 0.66 | 62 | 0.60 | 1.1 | 1.2 | 1.2 | 0.62 | 0.58 |
| Vl=50 < Vs=30 | 1.7 | 0.88 | FLR | --- | --- | 5.5 | 2.2 | 0.69 | 62 | 0.62 | 1.1 | 1.2 | 1.1 | 0.60 | 0.59 |
| Vl=80 < Vs=30 | 2.7 | 0.88 | FLR | 25.3 b | 0.033 | 5.6 | 2.2 | 0.72 | 61 | 0.63 | 0.89 | 0.89 | 1.1 | 0.78 | 0.78 |
| wNaCls=12.0%≅260 gs/Lmix |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vl=30 < Vs=80 | 0.33 | 0.74 | DLR | 79.6 a | 0.41 | 0.57 | 0.69 | 0.87 | 20 | 0.076 | 0.11 | 0.39 | 8.9 | 6.2 | 1.8 |
| Vl=30 < Vs=50 | 0.53 | 0.74 | DLR | 79.0 a | 0.63 | 0.92 | 0.70 | 0.83 | 20 | 0.12 | 0.24 | 0.63 | 5.5 | 2.8 | 1.1 |
| Vl=50 < Vs=80 | 0.55 | 0.74 | DLR | --- | --- | 1.9 | 2.3 | 0.73 | 42 | 0.26 | 0.54 | 1.4 | 2.7 | 1.3 | 0.50 |
| Vl≅42 < Vs=50 | 0.74 | 0.74 | FLR | --- | --- | 2.2 | 1.7 | 0.72 | 37 | 0.30 | 0.88 | 1.6 | 2.3 | 0.79 | 0.43 |
| Vl=Vs=30 | 0.88 | 0.74 | FLR | --- | --- | 1.9 | 0.87 | 0.75 | 29 | 0.26 | 0.78 | 0.99 | 2.7 | 0.89 | 0.71 |
| Vl=Vs=50 | 0.88 | 0.74 | FLR | --- | --- | 2.6 | 2.0 | 0.71 | 42 | 0.36 | 0.98 | 1.4 | 1.9 | 0.71 | 0.51 |
| Vl=Vs=80 | 0.88 | 0.74 | FLR | --- | --- | 2.9 | 3.6 | 0.68 | 50 | 0.40 | 0.96 | 1.5 | 1.7 | 0.72 | 0.46 |
| Vl=80 < Vs=50 | 1.4 | 0.74 | FLR | --- | --- | 3.1 | 2.3 | 0.74 | 45 | 0.42 | 0.74 | 0.81 | 1.6 | 0.94 | 0.86 |
| Vl=50 < Vs=30 | 1.5 | 0.74 | FLR | 30.9 b | 0.44 | 2.8 | 1.3 | 0.80 | 39 | 0.38 | 0.66 | 0.69 | 1.8 | 1.1 | 1.0 |
| Vl=80 < Vs=30 | 2.3 | 0.74 | FLR | 29.8 b | 0.55 | 3.0 | 1.4 | 0.83 | 40 | 0.41 | 0.56 | 0.57 | 1.7 | 1.3 | 1.2 |

(a) Feed-side threshold temperature, Tthres_s

(b) Distillate-side threshold temperature, Tthres_l

(a)						     (b)

Figure 8 – Evolution of the instantaneous a) GOR and b) SECth, for Csmax=120 gs/Lmix. Batch operating model.

(a)						     (b)

Figure 9 – Evolution of the instantaneous a) GOR and b) SECth, for Csmax=260 gs/Lmix. Batch operating model.

From these figures, we can see that the gain in the thermal efficiency (increase of GOR and reduction of SECth) when using the HX-2 is related to the overshooting in the instantaneous GOR and undershooting in the instantaneous SECth, which occur when the mixing tank is being emptied and refilled in each cycling process of the batch (or any other) operation mode.

This last statement can be verified in the following figures (Figure 10a-f), where the temporal evolution of the main DCMD system parameters (mixture salt concentration in the mixing tank - Csmix, volumetric flow rate of the produced distillate - Vjwl, mixture volume in the mixing tank - Vmix, and mixture temperature in the mixing tank - Tmix, volumetric flow rate of the brackish water well - Vwell and rejection fraction, λ) for the different desalination system operating modes (batch, semi-batch, volume-controlling (V-Control), continuous and steady-state) is illustrated.

Some particularities related to each of the operating modes are clearly identified in these figures. For example, for the batch operation mode, the mixture salinity varies between the brackish water well (Cswell ≈35 gs/Lmix) and the specified maximum salinity (Csmax=120 gs/Lmix) over each cycle (Figure 10a) as the distillate water is produced. A similar behavior is observed for the volumetric flow rate of the produced distillate, Vjwl, varying between 2.596 L/min and 2.256 L/min, respectively, according to Figure 10b, as the mixture salinity is continuously increased. The same behavior is seen for the semi-batch operating mode, although at a much lower frequency than in batch mode. In the volume-controlling operating mode (V-Control), these variables also oscillate between the above cited limits, however intermediary oscillations (in the salinity and the produced distillate) are present between each cycling period, due to the restrictions in the mixture tank volume (the minimum and maximum limits allowed, 60 L and 140 L, respectively).

The cycling time of each unsteady-state operating regime can be identified through the steep fall in the volume of the mixture tank (Figure 10c), which happens when the mixture is flushed-out of the tank, or through the sharp rise of the rejection fraction (Figure 10e). These behaviors are always accompanied by corresponding variations in the mixture temperature in the mixing tank (Figure 10d) and volumetric flow rate of the brackish water (Figure 10f). According to these figures, the cycling time for the batch mode is about 1789 s (≅ 30min) while for the V-control and semi-batch modes are 4800 s (≅ 1h20min) and 6090 s (≅ 1h41min), respectively.

It is interesting to observe that although the system is emptied and refilled with fresh brackish water from the well, at Twell=25 oC, the temperature of the mixture at the mixing tank does not reach this level at each cycling time because of the thermal inertia of the system, since the emptying and refilling process adopted in the present work is made in a sequential way, that is, the tank is continuously emptied until a pre-defined minimum volume is attained and then it is continuously refilled to the original volume, Vmix0=100 L.

(a)						     (b)

(c)						     (d)

(e)						     (f)

Figure 10 – Dynamic evolution of the instantaneous desalination system parameters for different operating modes and Csmax=120 gs/Lmix:  a) Csmix, b) Vjwl, c) Vmix, d) Tmix, e) λ, and f) Vwell.

Finally, in an attempt to identify the most appropriate operating mode of the DCMD desalination system, whether from the point of view of energy efficiency or water recovery, some comparisons are performed and illustrated in the Figs. 12 and 13, where the averaged specific thermal energy cost, SECth, and water recovery ratio, RR, are displayed, respectively.

The black bars (with gradient and solid filling) are the results for scenarios whose maximum salt concentrations are Csmax=120 gs/Lmix and Csmax=260 gs/Lmix, respectively. This means that in the unsteady-state batch, V-control, and semi-batch modes, the salinity of the mixture in the mixing tank can vary between the brackish water well salinity to the value defined to Csmax. For the steady-state modes, for the desalination system to operate continuously in one of the values set to Csmax, the final brine or the brackish water (or brine) from the well must have that level of salinity, so that Csoper= Csmax.

Figure 11 – Effect of the system operation mode on the averaged specific thermal energy cost, SECth, for different mixture salinity.

The blue bars appearing in the steady-state continuous mode represent the scenarios where Csoper= Csmax (66 gs/Lmix or 97 gs/Lmix), which are the average values of the system salinity when operating in the batch mode. In the steady-state operation scenario, as long as the brine from the DCMD module is rejected from the system, a saline stream with specified salinity (green bars: 66 or 97 gs/Lmix, black bars: 120 or 260 gs/Lmix) is continually pumped to the system at the predefined system flow rate (50 L/min).

Finally, the red bars represent two scenarios representing the salinity-control mode, when the concentration of the mixture in the mixing tank is limited to vary from 40 to 97 gs/Lmix  and from 66 to 120 gs/Lmix for the gradient- and solid-filled bars, respectively.

Figure 12 – Effect of the system operation mode on the water recovery ratio, RR, for different mixture salinity.

Overall, according to these figures, one must seek a balance between the specific energy cost and the water recovery ratio, since there exist, for the same maximum operational salinity, a trade-off between these metrics, i.e., you can't have a low specific thermal energy cost with high water recovery ratio at the same time.

When it comes to the energy cost, and for the same limiting mixture salinity, the best operating mode is the batch, followed by the semi-batch, steady-state, and continuous modes, as indicated in the literature (Swaminathan and Lienhard, 2018). On the other hand, when it comes to water recovery, Figure 12 reveals that the best choice is the semi-batch operating mode, followed by the V-control and batch modes, while the steady-state operating mode is prohibitive, unless the system is of the type analyzed by Sampaio (2022), in which there is no problem in totally rejecting the retentate that leaves the desalination module.

The behavior of the V-control operating mode in handling the scenario with intermediary salt concentration is especially interesting. Figure 12 shows that for Csmax=120 gs/Lmix, the system operating in the V-control model will produce more distillate water (RR=76.4 Ldist/Lwell against  RR=76.2 Ldist/Lwell) for the same input of brackish water from the well at the same energetic cost (SECth=0.40) when compared to the semi-batch operating mode. It is important to mention that this gain can be negligible since no energy costs related to control and intermittent pumping have been taken into account in this work.

Figure 12 also reveals that operating the desalination unit in a continuous way under a salinity level equivalent to the average of the salinity of the batch mode (blue bars) can present a lower energy cost than that of the batch mode at the expense of lower recovery ratio though.

Figure 13 shows the potential of each operating mode in producing distillate water when the desalination system is operated for 8h. In general, the desalination system can produce up to 1,200 L of distillate water running in any of the operating modes for the lower salinity level. On the other, when the salinity increases, it impacts negatively, especially for the steady-states modes (continuous and steady-state), when only about 750 L of distillate water is produced.

Figure 13 – Effect of the system operation mode on the produced volume of distillate water, for different mixture salinity.

Therefore, if the desalination system must be implemented in areas with hydrologic seasonal stress, as the semi-arid region of the Brazilian northeastern, the results point out that the semi-batch could be the preferred choice for brackish water desalination, since the main interest is in saving salt water from the well. Of course, the cost related to pumps and other intrinsic accessories for each operating modes must be considered in a more general thermo-economic analysis.

- Conclusions

The present work developed a mathematical modeling and computer simulation of a DCMD desalination system with double heat recovery from the distillate stream and the feed stream, together with total or partial recirculation of the retentate. The main goal was to employ the developed code to identify the best scenarios for operation of the system in terms of maximizing its thermal efficiency at the same time minimizing the use of brackish water from wells, seeking optimized production of the amount of distilled water.

In general, the following conclusions can be summarized from the present results:

- heat recovery from the distillate stream already produces a significant increase in the energy efficiency of the desalination system, being a mandatory condition for operation of DCMD-based systems;

- use of an additional heat exchanger to recover heat from the retentate stream also improves the energy efficiency of the process, especially when the system operates in the FLR condition with mass flow rate near the critical mass flow rate;

- peaks in GOR values and lows in SEC values can be observed when the well and an external heat exchanger coupled to its stream operate simultaneously, so double heat recovery and partial recirculation of the retentate play an important role in the competitiveness of the membrane distillation process compared to other forms of desalination, especially in long operation processes;

- the amount of heat supplied to the system is strongly impacted by the rejection fraction of the retentate stream;

- the system operating mode has a strong impact on the energy consumption and water recovery of the DCMD desalination system.

- The lowest specific thermal energy cost is associated with the batch operation mode of the desalination plant, however, in regions with hydrologic seasonal stress, the results point out that the semi-batch could be the preferred choice for brackish water saving. Therefore, its choice must be based on information as where the system has to be implanted and what are the availability of energy and brackish water salinity.

It should be clear, however, that to be conclusive a more in-depth study involving environment and economic aspects should be carried out. Extension to other desalination MD methods, like AGMD and V-AGMD, are currently in development, in order to simulate the entire experimental TRL-6 desalination unit installed in the LabMEMS/UFRJ, which operates in a controlling method based on the volume of the collector tank.

- References

Alkhudhiri, A., Darwish, N., Hilal, N. Membrane distillation: A comprehensive review. Desalination, 287, pp. 2-18, 2012.

Al-Obaidani, S., Crucio, E, Macedonio, F., Profio, G.D., Al-Hinai H., Drioli, E. Potential of membrane distillation in seawater desalination: thermal efficiency, sensitivity study and cost estimation. J. Memb. Sci., 323 (1), pp. 85-98, 2008.

Beeckman, J. Mathematical description of heterogeneous materials. Chem. Eng. Sci., 45, pp. 2603-2610, 1990.

Bruggeman, D.A.G. Calculation of Different Physical Constants of Heterogeneous Substances I: Dielectric Constant and Conductivity of Media of Isotropic Substances. Ann. Phys. Series-5, 24, pp. 636-664, 1935.

Charcosset, C. A review of membrane processes and renewable energies for desalination. Desalination, 245, pp. 214-231, 2009.

Christie, K.S.S., Horseman, T., Shihong, L. Energy efficiency of membrane distillation: simplified analysis, heat recovery and the use of waster-heat. Environment International, 138, 2020, 105588.

Chung, S., Seo, C.D., Lee, H., Choi, J.H., Chung, J. Design strategy for networking membrane module and heat exchanger for direct contact membrane distillation process in seawater desalination. Desalination, 349, pp. 126-135, 2014.

Curcio, E., Drioli, E. Membrane distillation and related operations – a review. Dep. Purif. Rev., 34, pp. 3586, 2005.

Curcino, I.V., Costa Júnior, P.R.S., Cárdenas Gómez, A.O., Peñaranda Chenche, L.E., Lima, J.A., Naveira-Cotta, C.P., Cotta, R.M. Analysis of effective thermal conductivity and tortuosity modeling in membrane distillation simulation, Micro and Nano Engineering, 17, 100165, 2022.

Drioli, E., Ali, A., Macedonio, F. Membrane distillation: Recent developments and perspectives. Desalination, 356, pp. 56-84, 2015.

Duong, H.C., Cooper, P., Nelemans, B., Cath, T.Y., Nghiem, L.D. Optimising thermal efficiency of direct contact membrane distillation by brine recycling for small-scale seawater desalination. Desalination, 374, pp. 1-9, 2015.

Elias-Kohav, T., Moshe, S., Avnir, D. Steady-state diffusion and reactions in catalytic fractal porous media. Chemical Engineering Science, 46 (11), pp. 2787-2798, 1991.

Elimech, M., Phillip, W.A. The future of seawater desalination: Energy, Technology and the Environment. Science, 333, pp. 712-717, 2011.

Fan, H, Peng, Y. Application of symmetric PVDF membranes in desalination and comparison of the VMD and DCMD processes. Chem. Eng. Sci., 79, pp. 94-102, 2012.

Fane, A.G., Schofield, R.W., Fell, C.J.D. The efficient use of energy in membrane distillation. Desalination, 64, pp. 231-243, 1987.

García-Payo, M.C., Izquierdo-Gil, M.A. Thermal resistance technique for measuring the thermal conductivity of thin microporous membranes. Journal of Physics D: Applied Physics, 37(21), pp. 3008–3016, 2004, https://doi.org/10.1088/0022-3727/37/21/011.

Ghanbarian, B., Hunt, A.G., Ewing, R.P., Sahimi, M. Tortuosity in porous media: a critical review. Soil Sci. Soc. Am. J., 77, pp. 1461-1477, 2013.

Gilron, J., Song, L.M., Sirkar, K.K. Design of cascade of crossflow direct contact membrane distillation. Ind. Eng. Chem. Res., 46 (8), pp. 2324-2334, 2007.

Gonzalez-Bravo, R., Elsayed, N.A., Ponce-Ortega, J.M., Napoles-Rivera, F., El-Halwagi, M.M. Optimal design of thermal membrane distillation systems with heat integration with process plants. Appl. Therm. Eng., 75, pp. 154-166, 2015.

Gostoli, C., Sarti G.C. Low temperature distillation through hydrophobic membrane. Sep. Sci. Technol., 22 (2&3), pp. 852-872, 1998.

Guan, G., Wang, R., Wicaksana, F., Yang, X., Fane, A.G. Analysis of membrane distillation crystallization system for high salinity brine treatment with zero discharge using Aspen Flowsheet Simulation. Ind. Chem. Res., 51, pp. 13405-13413, 2012.

Guan, G., Yang, X. Wang, R., Fane, A.G. Evaluation of heat utilization in membrane desalination system integrated with heat recovery. Desalination, 366, pp. 80-93, 2015.

Guan, G., Yang, X., Wang, R., Field, R., Fane, A.G. Evaluation of hollow fiber-based direct contact and vacuum membrane distillation systems using aspen process simulation. Journal of Membrane Science, 464, pp. 127-139, 2014.

Hausmann, A., Sanciolo, P., Vasiljevic, T., Weeks, M., Duke, M. Integration of membrane distillation into heat paths of industrial processes. Chem. Eng. J., 211-212, pp. 378–387, 2012.

He, F., Gilron, J., Sirkar, K.K. High water recovery in direct contact membrane distillation using a series of cascades. Desalination, 323, pp. 48-54, 2013.

IMSL Fortran Math Library. Perforce/Rogue Wave Software, Minneapolis, 2022.

Ismail, M.S., Mohamed, A.M., Poggio, D., Pourkashanian, M. Direct contact membrane distillation: A sensitivity analysis and an outlook on membrane effective thermal conductivity, Journal of Membrane Science, vol. 624, p. 119035, 2021.

Khayet, M.S., Godino, P., Mengual, J.I. Theory and experiments on sweeping gas membrane distillation. J. Membr. Sci., 165, pp. 261-272, 2000.

Khayet, M.S., Matsuura, T. Membrane distillation: principles and applications, 2011.

Khayet, M.S., Matsuura, T., Mengual, J.I., Qtaishat, M. Design of novel direct contact membrane distillation, membranes. Desalination, 192 (1-3), pp. 105-111, 2006.

Kim Woo-Ju, Campanella, O., Heldman, D.R. Predicting the performance of direct contact membrane distillation (DCMD): Mathematical determination of appropriate tortuosity based on porosity. Journal of Food Engineering, 294, 110400, 2021.

Kim, A.S. Cylindrical cell mode for direct contact membrane distillation (DCMD) of densely packed hollow fibers, Journal of Membrane Science, 455, pp. 168-186, 2014.

Kim, Y.D., Thu, K., Choi, S.H. Solar-assisted multi-stage vacuum membrane distillation system with heat recovery unit. Desalination, 367, pp. 161-171, 2015.

Kurokawa, H., Sawa, T. Heat recovery characteristics of membrane distillation. Heat Transf. Jpn. Res., 25 (3), pp. 135-150, 1996.

Lawson, K.W., Lloyd, D.R. Membrane distillation. Journal of Membrane Science, 124 pp. 1-25, 1997.

Lee, H.Y., He, F., Song, L.M., Sirkar, K.K. Desalination with a cascade of crossflow hollow fiber membrane distillation devices integrated with a heat exchanger. AICHE J., 54 (7), pp. 1780-1795, 2011.

Li, Z., Peng, Y., Dong, Y., Han, H., Chen, P., Qiu, L. Jiang, Q. Effects of thermal efficiency in DCMD and the preparation of membranes with low thermal conductivity. Appl. Surf. Sci., 317, pp. 338-349, 2014.

Li, B., Sirkar, K.K., Novel Membrane and Device for Direct Contact Membrane Distillation-Based Desalination Process. Ind. Eng. Chem. Res. 43, pp. 5300-5309, 2004.

Lin, S., Yip, N.Y., Elimelech, M. Direct contact membrane distillation with heat recovery: thermodynamic insights from module scale modeling. Journal of Membrane Science, 453, pp. 498-515, 2014.

Lisboa, K. M., Moraes, D. B., Naveira-Cotta, C. P., Cotta, R. M. Analysis of the membrane effects on the energy efficiency of water desalination in a direct contact membrane distillation (DCMD) system with heat recovery. Applied Thermal Engineering, 182, 2021,116063.

Lokare, O.R., Tavakkoli, S., Khanna, V., Vidic, R.D. Importance of feed recirculation for the overall energy consumption in membrane distillation systems. Desalination, 428, pp. 250-254, 2018

Lu, Y., Chen, J. Integration design of heat exchanger networks into membrane distillation systems to save energy. Ind. Eng. Chem. Res., 51, pp. 6798-6810, 2012.

Mackie, J., Meares, P. The diffusion of electrolytes in a cation-exchange resin membrane I. Theoretical. Proc. Roy. Soc. Lond. Math. Phys. Sci., 232 (1191), pp. 498-509, 1955.

Maheswari, K.S., Kalidasa Murugavel, K., Esakkimuthu, G. Thermal desalination using diesel engine exhaust waste heat - an experimental analysis. Desalination, 358, pp. 94-100, 2015.

Phattaranawik, J., Jiraratananon, R., Fane, A.G. Heat transport and membrane distillation coefficients in direct contact membrane distillation. Journal of Membrane Science, vol. 212(1–2), pp. 177–193, 2003.

Sampaio, P.A.B., Computational model and simulation of DCMD desalination systems with heat recovery. Desalination, 533, 2022, 115769.

Schneider, K., Hölz, W., Wollbeck, R. Membranes and modules for transmembrane distillation. J. Membr. Sci., 39 (1), pp. 25-42, 1988.

Sharqawy, M.H., Lienhard, J.H., Zubair, S.M. The thermophysical properties of seawater: A review of existing correlation and data. Desalin. Water Treat., 16, pp. 354-380, 2010.

Sparrow, B.S. Empirical equations for the thermodynamic properties of aqueous sodium chloride. Desalination, 159, pp. 161-170, 2003.

Swaminathan, J., Lienhard, V.J.H.  Design and operation of membrane distillation with feed recirculation for high recovery brine concentration. Desalination, 445, pp. 51-62, 2018.

Swaminathan, J., Chung, H.W., Warsinger, D.M., Lienhard V., J.H. Energy efficiency of membrane distillation up to high salinity: Evaluating critical system size and optimal membrane thickness. Applied Energy, 211, pp. 715–734, 2018.

Veiga, H.P. Encontros Temáticos - Saneamento Ambiental: Desafios e Soluções (in portuguese). ABES-RS, Ministério do Meio Ambiente, Porto Alegre/RS, Brazil, 2017.

Wang, K.Y., Chung, T.S., Gryta, M. Hydrophobic PVDF hollow fiber membranes with narrow pore size distribution and ultra-thin skin for the fresh water production through membrane distillation. Chemical Engineering Science, 63(9), pp. 2587-2594, 2008.

Wang, K.Y., Foo, S.W., Chung, T.S. Mixed matrix PVDF hollow fiber membranes with nanoscale pores for desalination through direct contact membrane distillation. Ind. Eng. Chem. Res., 48, pp. 4474-4483, 2009.

Wirth, D., Cabassud, D. Water desalination using membrane distillation: comparison between inside/out and outside/in permeation. Desalination, 147(1-3), pp. 139-145, 2002.

Yang, X. Wang, R., Shi., L., Fane, A.G., Debowski M. Performance improvement of PFDV hollow fiber-based membrane distillation process. Journal of Membrane Science, 369, pp. 437-447, 2011.

Yao, K., Qin, Y., Yuan, Y. A continuous-effect membrane distillation process based on hollow fiber AGMD module with internal latent-heat recovery. AICHE J., 59, pp. 1278-1297, 2013.

Zhang, J., Li, J., Duke, M., Xie, Z., Gray, S. Performance of asymmetrical hollow fiber membranes in membrane distillation under various configurations and vacuum enhancement. J. Membr. Sci., 362, pp. 517-528, 2010.

Zhang, Y., Peng, Y., Ji, S., Li, Z., Chen, P. Review of thermal efficiency and heat recycling in membrane distillation processes. Desalination, 367, pp. 223-239, 2015.